Instillation of your new Performance CDI

Once you remove your old cdi the following steps are recommended prior to instillation.

- Get some contact cleaner and clean the contacts of the old CDI plugs

- Use die-electric grease to ensure it is lubed and kept clean and seals

- Check the grounds on your frame and ensure they are solid. I recommend taking the ground off, sanding the area it connects, re-bolting it down and then touching up with clear paint. This will seal the frame, and ensure ground is solid.

- Clean connectors from stator to harness like the CDI plug and lube as well.

- Sparkplug Coil connectors: check to make sure they are not loose. If so, use a pair of pliers and squeeze down a little then re-plug in. This tightens the connections. Clean and lube as well.

- Coil wire may have corrosion where the plug boot connects. Unscrew plug boot, trim off about 1/4 to 1/3 “ off the end and screw the boot back on.